인터페이스 명세서  (Interface Specification)

# 1. ROS2 Topic 인터페이스

| Topic | Dir | Message Type | Payload | Consumer | QoS |
| --- | --- | --- | --- | --- | --- |
| /audio/raw | Sub | sensor_msgs/Audio | 16kHz mono PCM | Wake, VAD | BEF/Vol/1 |
| /wake/detected | Pub | std_msgs/Bool | true on detect | VAD | REL/Vol/1 |
| /vad/segment | Pub | rover_msgs/AudioSegment | speech bounds | STT | REL/Vol/1 |
| /stt/text | Pub | std_msgs/String | recognized text | Intent | REL/Vol/1 |
| /intent/result | Pub | rover_msgs/Intent | intent + confidence | LLM/VLM | REL/Vol/1 |
| /vision/detections | Pub | rover_msgs/Detections | YOLO results | Intent | BEF/Vol/1 |
| /llm/response | Pub | std_msgs/String | LLM output | TTS | REL/Vol/1 |
| /vlm/result | Pub | std_msgs/String | VLM output | LLM | REL/Vol/1 |
| /tts/cmd | Pub | std_msgs/String | text to speak | Host | REL/Vol/1 |
| /cmd_vel | Pub | geometry_msgs/Twist | velocity cmd | Host | REL/Vol/1 |
| /odom | Sub | nav_msgs/Odometry | position estimate | Nav2 | BEF/Vol/1 |
| /heartbeat | Pub | std_msgs/Header | 1Hz alive signal | Host | REL/Vol/10 |

# 2. Service 인터페이스

| Service | Provider | Request Type | Response | Purpose |
| --- | --- | --- | --- | --- |
| /llm/generate | llm_runner | rover_srv/Generate | text in, stream out | LLM 응답 |
| /vlm/analyze | vlm_runner | rover_srv/AnalyzeImage | image + query | VLM 분석 |
| /nav2/navigate | nav2 | nav2_msgs/NavigateToPose | goal pose | 경로 안내 |
| /arm/move | moveit | moveit_msgs/MoveGroup | target pose | 로봇팜 이동 |
| /system/swap_model | model_manager | rover_srv/SwapModel | model name | 모델 전환 |

# 3. Action 인터페이스

| Action | Server | Goal/Result Type | Feedback |
| --- | --- | --- | --- |
| /nav2/navigate_to_pose | Nav2 | nav2_msgs/NavigateToPose | 진행률/남은 거리 |
| /arm/follow_joint_trajectory | MoveIt | control_msgs/FollowJointTrajectory | 구동 완료/오류 |
| /tts/speak | TTS | rover_action/Speak | 발화 완료 신호 |

# 4. 커스텀 메시지 정의

| Message | Field | Description |
| --- | --- | --- |
| rover_msgs/Intent | string intent | 분류된 의도 (MOVE, PICK, DESCRIBE...) |
|  | float64 confidence | 신뢰도 (0.0 ~ 1.0) |
|  | string target | 대상 객체 이름 |
|  | geometry_msgs/Pose target_pose | 대상 3D 위치 (optional) |
| rover_msgs/Detection | int64 class_id | YOLO 클래스 ID |
|  | string class_name | 클래스 이름 |
|  | float64 confidence | 탐지 신뢰도 |
|  | sensor_msgs/RegionOfInterest bbox | 바운딩 박스 |
|  | geometry_msgs/Point3D position | 3D 위치 (depth 기반) |