GPU/메모리 예산표  (Resource Budget)

# 1. RAM 8GB 배분

| 컴포넌트 | GPU MB | RAM MB | Peak MB | 비고 |
| --- | --- | --- | --- | --- |
| OS/Kernel | - | 1000-1500 | 1500 | 시스템 기본 |
| ROS2 DDS | - | 200-300 | 300 | SHM + 노드 |
| Wake Detector | - | 50 | 50 | Porcupine |
| VAD | - | 30 | 30 | Silero ONNX |
| STT | 200 | 100 | 150 | moonshine |
| TTS | - | 80 | 100 | Piper |
| YOLO11n | 150 | 200 | 250 | TensorRT |
| OCR | 100 | 150 | 200 | PaddleOCR |
| Intent Router | - | 50 | 50 | Embedding |
| State Manager | - | 30 | 50 | SQLite |
| LLM 1.5b | 800 | 400 | 1200 | llama.cpp |
| LLM 0.5b | 400 | 200 | 600 | llama.cpp |
| VLM 1.8b | 1200 | 800 | 2000 | moondream |
| Nav2 Stack | - | 300 | 400 | Planner + Controller |
| SLAM | - | 150 | 200 | slam_toolbox |

# 2. GPU 사용량

| GPU Task | VRAM MB | Compute % | Notes |
| --- | --- | --- | --- |
| YOLO11n Inference | 150 | 15% | 30 FPS @ 640x480 |
| STT Encoder | 200 | 10% | moonshine-tiny |
| LLM 1.5b (q4_K_M) | 800 | 40% | llama.cpp CUDA |
| VLM 1.8b (q4) | 1200 | 60% | moondream vision |
| OCR Inference | 100 | 5% | PaddleOCR-lite |
| TensorRT Engine | 50 | - | Context overhead |

# 3. 시나리오 별 메모리

| 시나리오 | RAM | GPU | Status | 설명 |
| --- | --- | --- | --- | --- |
| Always-On Only | 2.2 GB | 250 MB | SAFE | YOLO + STT + TTS |
| + LLM 1.5b | 3.4 GB | 1050 MB | SAFE | 일반 대화 |
| + VLM (moondream) | 4.2 GB | 1450 MB | SAFE | 시각 분석 |
| + Nav2 + SLAM | 5.0 GB | 1450 MB | SAFE | 전소 기능 |
| Peak (Transition) | 6.2 GB | 2000 MB | WARN | 모델 전환 시 |
| Emergency Limit | 6.5 GB | - | DANGER | OOM 진입 전 |

# 4. OOM Score 설정

| Process | oom_score_adj | Priority | Action on OOM |
| --- | --- | --- | --- |
| host_bridge | -1000 | ABSOLUTE | Never kill |
| ros_robot_controller | -1000 | ABSOLUTE | Never kill |
| wake_detector | -300 | HIGH | Protected |
| vad_processor | -300 | HIGH | Protected |
| tts_server | -300 | HIGH | Protected |
| state_manager | -400 | HIGH | Protected |
| yolo_detector | -100 | MEDIUM | Protected |
| llm_runner | 800 | SACRIFICE | First to kill |
| vlm_runner | 700 | SACRIFICE | Second to kill |