Failure Mode & Effects Analysis  (FMEA)

# FMEA Table

| Failure | Cause | Effect | Detection | Mitigation | S | P |
| --- | --- | --- | --- | --- | --- | --- |
| OOM Kill | Memory leak | AI process death | Memory monitor | LLM kill, restart | 5 | 3 |
| Brain hang | Deadlock/infinite loop | No response to voice | Heartbeat timeout | Watchdog restart | 5 | 2 |
| STT fail | Audio driver issue | Cannot understand user | VAD timeout | TTS retry prompt | 3 | 3 |
| YOLO drop | GPU overload | Object not detected | FPS monitor | Reduce resolution | 3 | 2 |
| Nav failure | Localization lost | Wrong navigation | AMCL covariance | Re-localize | 4 | 2 |
| LLM hallucinate | Model limitation | Wrong action | Intent confidence | Confirm with user | 3 | 4 |
| VLM timeout | Thermal throttling | Scene not analyzed | Deadline check | Fallback to YOLO | 2 | 3 |
| E-stop fail | Hardware fault | Cannot stop robot | Hardware loop | Mechanical brake | 5 | 1 |
| State loss | DB corruption | Context reset | DB checksum | Empty context | 2 | 2 |
| Thermal shutdown | Fan failure | System halt | Temp sensor | Reduce power mode | 4 | 2 |
| Gripper slip | Object weight/texture misjudge | Object drop | Force feedback | Re-grasp + slower approach | 3 | 3 |
| SLAM drift | Long operation accumulation | Path error grows | Pose covariance monitor | Periodic re-localize | 3 | 2 |
| Mic array channel fail | Hardware degradation/vibration | Direction estimation fail | Per-channel health check | Fallback to mono mode | 3 | 2 |
| USB disconnect | Vibration/contact fault | Camera/mic loss | udev rule monitor | Auto-reconnect + TTS alert | 4 | 2 |
| Arm collision | IK solution error/obstacle | Servo damage/object damage | Joint torque monitor | Emergency stop + retract | 4 | 2 |
| Servo overheat | Continuous heavy load | Servo shutdown | Temp sensor per servo | Cool-down pause + alert | 3 | 2 |

# Risk Priority Matrix

| Severity \u2192 | 5 (Critical) | 4 (High) | 3 (Medium) |
| --- | --- | --- | --- |
| 5 (Very High) | CRITICAL | CRITICAL | HIGH |
| 4 (High) | CRITICAL | HIGH | HIGH |
| 3 (Medium) | HIGH | MEDIUM | MEDIUM |
| 2 (Low) | MEDIUM | LOW | LOW |