Intent → Action 매핑 테이블  (Intent to Action Mapping)

# Intent 분류 및 처리

| Intent | Condition | Action | Executor | Fallback |
| --- | --- | --- | --- | --- |
| MOVE | target location | Navigate to pose | Nav2 | Ask clarification |
| PICK | object detected | Arm pick sequence | MoveIt | Cannot reach |
| PLACE | holding object | Arm place sequence | MoveIt | No object held |
| FOLLOW | person detected | Person following | Nav2 | Lost target |
| DESCRIBE | scene request | VLM analyze | VLM | YOLO fallback |
| COUNT | objects in view | YOLO count | YOLO | No objects |
| IDENTIFY | label visible | OCR read | OCR | Cannot read |
| STOP | any time | Emergency stop | Host | Mechanical brake |
| GREET | wake word | TTS greeting | TTS | None |
| QUESTION | general query | LLM answer | LLM | Web search |

# Confidence 처리 룰

| Confidence | Range | Action | Model |
| --- | --- | --- | --- |
| HIGH | >= 0.85 | Execute immediately | Rule-based |
| MEDIUM | 0.60 - 0.84 | LLM confirmation | qwen2.5-0.5b |
| LOW | < 0.60 | Ask rephrase | TTS prompt |
| AMBIGUOUS | multi candidates | VLM disambiguate | moondream |

# Fallback 처리

| Failure | Condition | Fallback Action |
| --- | --- | --- |
| Object not found | YOLO empty | “찾지 못했어요” + VLM scan |
| Multiple matches | YOLO count > 1 | “어떤 것인가요?” + spatial query |
| Navigation fail | Planner timeout | “가지 못해요” + stop |
| Arm reach fail | IK timeout | “닿지 못해요” + closer approach |
| LLM timeout | > 3s TTFT | “생각중이에요” + retry |
| VLM unavailable | Mem < 2.5GB | YOLO-only mode |